# Tender text

# Motion/presence detector theMura S180 Slave B UP

Order No.: 			2060760

Features:

- Passive infrared motion/presence detector for wall mounting in flush-mounted box

- Without cover, suitable for many common switch ranges (with accessories), for details see www.theben.de/theMura

- Slave device for the extension of the detection area of theMura detectors

- Rectangular detection area 170°, max. 14 m x 17 m

- Area restriction in scope of delivery

- Integrated button (can be deactivated)

- Fully or semi-automatic operation, switchable (can be set at master)

- Staircase light function (can be set at master)

- Extended function (can be set at master)

- Ready for immediate use due to factory presetting

- Test mode for checking the detection area

Technical data:

Operating voltage: 		230 V AC

Frequency: 			50 - 60 Hz

Power consumption: 		approx. 0.4 W

Radial detection area: 		12 x 10 m

Tangential detection area: 	14 x 17 m

Installation height Mh: 		0.8 – 1.2 m

Protection rating: 		IP20

Operating temperature: 		-15 °C to +45 °C

Dimensions: 			85 x 85 mm, installation depth 37 mm

Installation: 			wall mounting in flush-mounted box size 1

Optional accessories:

### Cover theMura FED WH

### White colour, design to match Feller EDIZIOdue

### for details see www.theben.de/theMura

Order No.:			9070786

### Cover theMura LM WH

### White colour, design to match Legrand Mosaic

### for details see www.theben.de/theMura

Order No.:			9070787

### Cover theMura BJBSI WH

### White colour, design to match BJE balance SI

### for details see www.theben.de/theMura

Order No.:			9070788

### Cover theMura JAS WH

### White colour, design to match Jung AS 500

### for details see www.theben.de/theMura

Order No.:			9070789

### Cover theMura MMP WH

### White colour, design to match Merten M-Plan

### for details see www.theben.de/theMura

Order No.:			9070790

### Cover theMura GS WH

### White colour, design to match Gira Standard 55

### for details see www.theben.de/theMura

Order No.:			9070791

### Cover theMura BS WH

### White colour, design to match Berker S.1

### for details see www.theben.de/theMura

Order No.:			9070792

### Cover theMura Theben WH

### White colour, Theben-Design

### for details see www.theben.de/theMura

Order No.:			9070798