# Tender text

# Presence sensor PlanoSpot 360 DALI-2 S DE WH

Order No. white: 	2030190 (PlanoSpot 360 DALI-2 S DE WH)

Features:

- Passive infrared presence sensor DALI-2 for ceiling installation

- This device provides information about presence and brightness in the form of a DALI telegram in line with IEC 62386 Part 303/304.

- Square detection area 360°, moving up to 64 m2 (8 m x 8 m)

- 2 instances for presence - detection zone standard/reduced

- Optics can be tilted to adjust the detection area

- 3 mixed light measurements

- 4 instances for brightness - interior/centre/window/integral

- Easy to calibrate brightness measurement

- Test mode for checking function and detection area

- Configurable detection sensitivity

- Can be configured via DALI bus or remote control

- The «theSenda S» or «theSenda B» user remote controls can be used to dim or switch the lamps, to control the blinds, or to execute additional functions via the higher-level control.

- Flat design with interchangeable bezel frame in different colours

- This device corresponds to EN 60669-2-5 and IEC 62386-101/103/303/304

Technical data:

Operating voltage: 		DALI (in accordance with IEC 62386-101): 10 V – 22.5 V

Power input: 			max. 10 mA

Stationary detection area: 	4.5 m x 4.5 m (IH 3 m)

Detection area walking: 		7.0 m x 7.0 m (IH 3 m)

Recommended installation height IH: 2.0 – 3.0 m/max. 3.5 m

Light measurement: 		approx. 10 – 10,000 lux

Protection rating: 		IP20

Operating temperature: 		0 °C to +50 °C

Dimensions: 			Ø 76 mm, installation depth 69 mm

Connection type:		Plug-in terminal / 0.5 - 1.5 mm2

Installation: 			Ceiling installation with springs

Installation: 			Surface mounting on ceilings possible with back box 75B (option)

Optional accessories:

### theSenda B universal remote control

User remote control with integrated lux meter.

Remote control for installers: The free theSenda Plug app (for iOS/Android) is easy to use for commissioning or adjusting the parameters. Parameter sets can be saved and named in a customer-specific way and exported for archiving, for instance via email.

Order No.: 			9070985

### theSenda P service remote control

Order No.: 			9070910

### theSenda S user remote control

Order No.: 			9070911

### Back box 75B

Order no. white: 		9070796 (back box 75B WH)

Order No. black: 		9070824 (back box 75B BK)

### Bezel frame PlanoCover 76

Order No black: 		9070977 (PlanoCover 76 BK)

Order No. silver: 		9070978 (PlanoCover 76 SR)