Rail mounted staircase time switch ELPA 6 with 10 functions, optimized for LED (ELPA 6 plus)

Rail mounted staircase time switch with 10 functions, width 17.5 mm, time range 0.5 - 20 Min., can be retriggered or prematurely switched off, switch-off pre-warning optimized for LED (DIN 18015-2) selectable, time range 60 Min., power surge switch function, continuous light function, 3 and 4 wire connection, zero point switching, max. 150 mA glow lamp load with electronic overload protection, 1 NO contact 16(16)A/230 V~.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: ELPA 6 plus

Order No.: 0060003

Unit: ST