4-channel upgrade module EM 4 top2

4-channel upgrade module for time switches TR 641 top2 RC, TR 642 top2 RC and TR 644 top2 RC; rail mounted housing; width 72 mm; 4 external sensors/switch input (an input for each channel); switching pre-selection; permanent ON/OFF; 4 LEDs for displaying switching status; zero phase switching; DuoFix spring terminal for 2 conductors; 2 two-way switch 16 A /250 V~ and 10 A /250 V~;

Deliver, assemble, connect up and set up

...

Make: Theben

Type: EM top2

Order no.: 6490104

Unit: ST