# Tender text for presence detectors

# Presence detector PlanoSpot 360 KNX DE WH / BK / SR

Passive infrared presence detector for ceiling installation; Flat design with interchangeable bezel frame in different colours; Automatic presence and brightness dependent control for lighting and HVAC; Square detection area 360° max. walking 64 m2 (8 x 8 m), max. sitting 20 m2 (4.5 x 4.5 m) switchable via remote control or parameter to walking 22 m2 (4.7 x 4.7 m), sitting 9 m2 (3 x 3 m); Manual alignment of the detection area (swivel mechanism);

Mixed light measurement with 3 directed light measurements for fluorescent lamps (FL/PL/ESL), suitable for halogen/incandescent lamps and LEDs;

Energy consumption approx. 8 mA / 9 mA with LED on; protection rating IP 20;

2 channels C1, C2 light with two light measurements and additional lighting channel C3 without influence of brightness; Switching or constant lighting control with 2 independent controls and standby functionality

(orientation light); Switching operation with dimmable lighting; Fully or semi-automatic device; Brightness switching or setpoint value adjustable in lux via parameter, object or remote control; Teach-in of brightness switching or setpoint value; Self-learning time delay adjustable via parameter, object or remote control; Reduction of time delay during brief presence (short-term presence); Manual override by means of telegram or remote control;

2 presence channels C4, C5, individually programmable; Switch-on delay and time delay adjustable; Room monitoring with selective motion detection

Adjustment of the room correction factor for calibrating the brightness measurement; Configurable sensitivity; Simple configuration of the energy-saving behaviour via "eco plus" function; Test mode to check function and detection area; Scenes; Extension of the detection area through parallel switching of several presence detectors (master/slave or master/master); Remotely configurable via remote control;

Installation in hollow ceilings with springs for ceiling thicknesses of 0.5 to 30 mm, ceiling cut-out Ø 68 mm, installation depth 69 mm; Ceiling installation possible with back box 75A;

Manufacturer: thebenHTS

Type: PlanoSpot 360 KNX DE..

Order No.: 2039100 (PlanoSpot 360 KNX DE WH, white)

2039101 (PlanoSpot 360 KNX DE BK, black)

2039102 (PlanoSpot 360 KNX DE SR, silver)

Unit: ST

### Accessories

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order no.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911

### Back box 75A WH

Optional

Order no.: 9070949

### Back box 75A BK

Optional

Order no.: 9070950

### Back box 75A SR

Optional

Order no.: 9070951