Clock thermostat with daily and weekly program, operating mode switch (RAM 722)

Electronic thermostat switchable for 24 hours or 7 day program; automatic temperature reduction at night, programmable every 5 minutes or 30 minutes; shortest switching time for weekly program 2 hours; day and night temperature settings from +10°C ... 30°C; operating mode switch; frost protection 6°C; party switch; indicator display; thermal circulation and operating point settings; housing colour: white; 1 two way switch 6A/250V~.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RAM 722

Order No.: 7220030

Unit: ST