Flush-mounted room thermostat, connection for temperature reduction, heating switch (RAM 746)

Room thermostat; can be combined with sub-frames (50x 50mm) from different switch manufacturers for area switch programs; 5 - 30°C; temperature reduction (approx. 4 K) option via external control (e.g. switch; timer); heating switch ON/OFF with indicator lamp; colour: pure white; NC contact 10 A/250 V~.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RAM 746

Order No.: 7460130

Unit: ST