Rail mounted time switch 1 channel, astronomic, 1 external input, 1 module (SELEKTA 175 top2)

1 channel rail mounted astronomical time switch with factory-set time; text-oriented user guidance in the display; width 17.5 mm; astronomic program switches off/on at sunrise and sunset; city database; automatic summer/winter time changeover; maximum offset +/-120 mins, weekly program plus, 54 memory locations; memory card/PC interface; elapsed-time counter for light sources; ON/OFF permanent; DuoFix screwless terminals; zero cross switching; PIN code; 10 year power reserve, NO contact 16A/250V~; sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: SELEKTA 175 top2

Order no.: 1750100

Unit: ST

Accessories:

OBELISK top2 programming set

Memory card, plug-in adapter (USB), software

For Windows 2000/XP/Vista/Win7/Win8

Order no.: 9070409

OBELISK top2 memory card, single

Order no.: 9070404