Rail mounted time switch, astronomic, 2 module (SEL 171 top2 RC)

Digital rail mounted astronomic time switch with factory-set time; text-oriented user guidance in the display; width 35 mm; astronomic program switches off/on at sunrise and sunset; city database; automatic summer/winter time changeover; maximum offset +/-120 mins, weekly program plus 3 holiday and bank holiday programs, 84 memory locations; memory card/PC interface; RC-DCF antenna interface; external control input with 5 functions; elapsed-time counter for light sources; ON/OFF permanent; DuoFix screwless terminals; zero cross switching; PIN code; 10 year power reserve, 1 two way switch 16A/250~; sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: SEL 171 top2 RC

Order no.: 1710100

Unit: ST

Accessories:

Terminal box cover plate

Order no.: 907 0 050

OBELISK top2 programming set

Memory card, plug-in adapter (USB), software

For Windows 2000/XP/Vista

Order no.: 907 0 409

OBELISK top2 memory card, single

Order no.: 907 0 404

top2 RC–DCF antenna

Order no.: 907 0 0011