Rail mounted digital time switch 60 min., segments, 3 TlE (SYN 151 d)

Rail mounted time switch without power reserve; width 52.5mm; 60 min. segment program; shortest switching time 37.5 seconds; switching pre-selection; manual switch with 3 positions. Continuous ON/AUTO/continuous OFF; switching status display; spring terminal for 2 conductors; 1 two way switch 10 A /250 V~; 230V / 50 Hz;sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: SYN 151 d

Order no.: 1510011

Unit: ST