# thePiccola P360-100 DE WH 1 channel mini presence detector

Passive-infrared presence detector for ceiling installation; automatic presence and brightness-dependent control for lighting; round 360° detection area, walking up to 50 m2 (Ø 8 m), installation height up to 4 m;

Mixed light measurement suitable for fluorescent lamps (FL/PL/ESL), halogen/incandescent lamps and LEDs;

Operating voltage 110 - 240 V AC / 50 - 60 Hz; energy consumption approx. 0.5 W, protection rating sensor IP 21, power supply unit IP 20; operating temperature -20 °C to +50 °C;

Channel A light: relay, 1 NO contact 240 V / 10 A, 2000 W or 1300 VA (140 µF); can be switched between fully automatic or semi-automatic operation; configurable brightness switching value approx. 5 – 1000 lux, teach-in function; time delay 30 s – 30 min; connection option for buttons for manual switching; pulse function for staircase light timer switch;

Ready for operation immediately thanks to factory presetting; can be configured remotely with remote control; test mode to check function and detection area; extension of detection area by master/slave switching;

Installation in hollow ceilings with springs, ceiling cut-out Ø 34 - 36 mm, installation depth 22 mm;

Make: Theben

Type: thePiccola P360-100 DE WH (white)

Order no.: 2090200

Unit: ST

### Accessories

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order no.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911