# Tender text for presence detectors

# Presence detector thePrema P360 KNX UP WH / GR

Passive-infrared presence detector for ceiling installation; automatic presence and brightness-dependent control for lighting and HVAC; square 360° detection area, walking 100 m2 (10 x 10 m), seated 64 m2 (8 x 8 m) at 3.5m installation height;

Mixed light measurement with 3 directed light measurements for fluorescent lamps (FL/PL/ESL), suitable for halogen/incandescent lamps and LEDs;

Energy consumption approx. 9 mA / 13 mA with LED on; protection rating IP 20, IP 40 when installed;

2 channels for light C1, C2 with two light measurements and an additional lighting channel C3 not affected by brightness; switched or constant light control with stand-by functionality (orientation light); switched mode with dimmable lighting; fully or semi-automatic brightness switching value or setpoint adjustable in lux via parameter, object or remote control; teach-in of brightness switching value or setpoint; self-learning time delay adjustable via parameter, object or remote control; reduction of time delay during brief presence (brief presence); manual override by means of telegram or remote control;

2 channels for presence C4, C5, individually programmable; switch-on delay and time delay adjustable; room monitoring with selective motion detection

Adjustment of the room correction factor for calibrating the brightness measurement; configurable sensitivity; simple configuration of the energy-saving behaviour via the "eco plus" function; test mode to check function and detection area; scenes; extension of detection area through parallel switching of several presence detectors (master/slave or master/master);

Installation depth 40 mm, ceiling installation in flush-mount socket or ceiling installation socket 73A, ceiling installation possible with surface-mount frame;

Manufacturer: thebenHTS

Type: thePrema P360 KNX UP..

Order no.: 2079000 (thePrema P360 KNX UP WH, white)

2079001 (thePrema P360 KNX UP GR, grey)

Unit: ST

### Accessories

### SendoPro 868-A management remote control

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual settings or complete configuration sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order no.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911

### Surface mount frame 110A WH

Optional

Order no.: 9070912

### Ceiling flush-mounting box 73A

Optional

Drill diameter 73 mm

Order no.: 9070917