# Tender text for presence detectors

# Presence detector theRonda P360 Slave UP WH / GR

Passive-infrared presence detector for ceiling installation; round 360° detection area, walking up to 452m2 (Ø 24m), installation height up to 10m, area restriction with covering clips (optional);

Operating voltage 110 - 230 V AC / 50 - 60 Hz; energy consumption approx. 0.1 W, Protection rating IP 20, in fitted state IP 54; operating temperature -15°C to +50°C;

Configurable sensitivity; ready for operation immediately thanks to factory presetting; can be configured remotely with remote control; test mode to check function and detection area; extension of detection area by master/slave or master/master switch;

Installation depth 37mm, ceiling installation in flush-mounted box size 1 or ceiling installation socket 73A, ceiling installation possible with back box 110A;

Manufacturer: thebenHTS

Type: theRonda P360 Slave UP.. (Can only be used with a master device)

Order no.: 2080030 (theRonda P360 Slave UP WH, White)

2080031 (theRonda P360 Slave UP GR, Grey)

Unit: ST

### Accessories

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order No.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911

### Back box 110A WH

Optional

Order no.: 9070912

### Back box 110A GR

Optional

Order no.: 9070913

### Ceiling installation socket 73A

Optional

Drill diameter 73mm

Order no.: 9070917

### Cover clip for area restriction

Optional

Order no.: 9070921