Rail mounted 365-day timer 1 channel, upgrade and RC capable (TR 641 top2 RC)

1-channel 365-day timer; RC: Time synchronisation via connection of external DCF or GPS antennas (additional positioning required for astronomical program with GPS, top2 GPS power unit required); 4-channel upgrade module (EM 4 top2) and LAN module (EM LAN top2) connection; rail mounted housing; width 52.5 mm, annual and astronomical programming; text-oriented operator guidance in the display; 800 memory locations; interface for OBELISK top2 memory card (PC programming); 6-year power reserve (replaceable lithium battery); automatic daylight savings time switchover; ON-OFF switching time; impulse program; cycle program; extensive annual clock functions; astronomical switching function (automatic calculation of sunrise and sunset times for the whole year); astronomical offset; astronomical impulse; switching pre-selection; delay timer; permanent ON/OFF/AUTO switching; integrated lapsed time counter; external sensors/switch input; holiday program; holiday program; 2 random programs; display lighting (can be shut down); PIN coding; zero phase switching; DuoFix spring terminal for 2 conductors; 1 two-way switch 16 A /250 V~; sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: TR 641 top2 RC

Order no.: 6410300

Unit: ST