Rail mounted 365-day time switch 4 channel, OBELISK interface (TR 644 top2)

4 channel annual time switch; rail mounted housing; width 72 mm, annual and astronomical programming; text-oriented operator guidance in the display; 800 memory locations; interface for OBELISK top2 memory card (PC programming); OBELISK memory card included; 6-year power reserve (replaceable lithium battery); automatic daylight savings time switchover; ON-OFF switching time; impulse program; cycle program; extensive annual clock functions; astronomical switching function (automatic calculation of sunrise and sunset times for the whole year); astronomical offset; astronomical impulse; switching pre-selection; delay timer; permanent ON/OFF/AUTO switching; integrated lapsed time counter; 4 external sensors/switch inputs (an input for each channel); holiday program; holiday program; 2 random programs; display lighting (can be shut down); PIN coding; zero phase switching; DuoFix spring terminal for 2 conductors; 2 two-way switch 16 A /250 V~ and 2 two-way switch 10A/250V~; sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: TR 644 top2

Order no.: 6440100

Unit: ST